Table of Contents

1.0 Gantt Chart	4

2.0 Project Budget	5

2.1 Total Cost (RM)	5

2.2 Network Hardware and Components	7

2.2.1 PC	7

2.2.2  Laptop	9

2.2.3 Router	10

2.2.4 Switch	11

2.2.5 Access Point	12

2.2.6 Server	13

2.3 Network Software and Configurations	14

2.3.1 Network Software	14

2.4 Cabling and Sockets Component	16

2.4.1 Cables	16

2.5 Main Hour Requirements	19

3.0 Network Topology	20

3.1 Logical Design	20

3.2 Physical Design	21

4.0 IPv6 Addressing Table	22

4.1 Lab1	22

4.2 Lab2	23

4.3 System Adminstrator office	23

4.4 Administrator Office	24

4.5 Wireless device	24

4.6 Server	24

4.7 Router	25

4.8 Classrooms	25

5.0 Router Configuration	26

5,1 IPv6 Address Configuration	26

5.2 DHCPv6 Configuration	29

5.3 OSPF configuration	31

6.0 Frame Relay Configuration	34

7.0 Access Control List Configuration	36

8.0 Server Configuration	37

8.1 Web Server Configuration	37

8.2 DNS Server Configuration	39

8.3 File Server Configuration	42

8.4 ISP Server Configuration	45

8.5 Cloud	46

9.0 Verify Connectivity	48

9.1 Ping(ICMP)	48

9.2 Web Browser	53

9.3 File Server	55

9.4 Access control	59

10 .0 Reference	61

# 1.0 Gantt Chart

# 2.0 Project Budget

## 2.1 Total Cost (RM)

| Network Hardware and Components | Quantity | Cost Per Price (RM) | Total Price (RM) |
| --- | --- | --- | --- |
| Dell OptiPlex 7010 MT (PC) | 64 | 1600 | 102400 |
| Dell Latitude E6430 (Laptop) | 1 | 1400 | 1400 |
| Cisco Catalyst 2960-24TT-L (Switch) | 7 | 1000 | 7000 |
| Cisco ISR 4331 (Router) | 4 | 8000 | 3200 |
| Cisco Aironet 1832i (Access Point) | 1 | 1000 | 1000 |
| Dell PowerEdge R710 (Server) | 4 | 3200 | 12800 |
| Network Software |  |  |  |
| Cisco Packet Tracer | - | Free | - |
| Windows Server 2025 Standard Edition | 1 | 3000 | 3000 |
| PRTG Network Monitor (commercial) | 1 | 6000 | 6000 |
| Cabling and Socketing Components |  |  |  |
| Cat6 RJ-45 Ethernet Cable (3m) | 75 | 10 | 750 |
| Cisco Smart Serial DCE/DTE Cable (3m) | 4 | 48 | 184 |
| NIM-2T | 5 | 1600 | 8000 |
| Total Cost (RM) |  |  | 145734 |

## 2.2 Network Hardware and Components

### 2.2.1 PC

Dell OptiPlex 7010 MT

The Dell OptiPlex 7010 Mini Tower is a durable and budget-friendly desktop PC designed for everyday use in educational labs and office environments. It provides sufficient computing power for routine tasks such as browsing, file management, and accessing network services.

| Features | Benefit |
| --- | --- |
| Intel Core i3 (3rd Generation) processor | Executes daily routine tasks efficiently with low power consumptions |
| Expandable memory (up to 16 GB DDR3) | Allows system upgrades to support future software requirement |
| 500GB HDD (SSD upgrade supported) | Provides large storage and faster performance when upgraded to SSD |
| Intel HD Graphis 2500 | Supports multimedia presentations without extra hardware cost. |
| Gigabit Ethernet interface | Ensures and high-speed network connectivity |
| Windows OS support | Compatible with a wide range of educational and office applications |

### 2.2.2  Laptop

Dell Latitude E6430

The Dell Latitude E6430 laptop represents mobile users who access the network through the wireless access point. It demonstrates how portable devices can participate in enterprise networks without physical cabling limitations. Its built-in wireless adapter allows dynamic association with the access point, supporting mobility within the coverage area. The dual capability of wired and wireless networking provides flexibility for testing different access methods and evaluating wireless network performance, authentication, and security behaviour.

| Feature | Benefit |
| --- | --- |
| Built-in Wi-Fi adapter | Enables wireless access through the access point |
| Portable design | Allows flexible usage in different physical locations |
| Battery powered | Can operate without continuous power connection |
| Ethernet port support | Allow both wired and wireless networking. |
| Business-grade construction | Provides durability for long-term use |

### 2.2.3 Router

Cisco ISR 4331

| Features | Benefits |
| --- | --- |
| IPv6 support and OSPFv3 routing | Enables modern network communication and automatic route learning, reducing manual configuration errors. |
| Multiple Gigabit Ethernet ports | Allows high-speed connections between LAN segments, switches, and servers. |
| Modular interface slots (supports NIM-2T) | Provides flexibility to add serial ports for WAN and Frame Relay simulation. |
| Access Control List (ACL) capability | Enhances network security by controlling which traffic is allowed or denied. |
| High processing performance | Ensures stable data forwarding even when many devices are connected simultaneously. |

The Cisco ISR 4331 is an enterprise-grade integrated services router designed for medium-sized networks such as campuses, laboratories, and branch offices. It provides reliable IPv6 routing, dynamic routing protocol support, WAN connectivity, and network security functions. In this project, the router acts as the core device that interconnects different departments, servers, and external networks.

### 2.2.4 Switch

Cisco Catalyst 2960-24TT-L

The Cisco Catalyst 2960-24TT-L is a Layer-2 access switch commonly deployed in enterprise networks to connect computers, servers, and network devices within the same local area network. It is used in this topology to provide wired connectivity for labs, offices, and server rooms.

| Features | Benefits |
| --- | --- |
| 24 Fast Ethernet ports | Supports large numbers of PCs in a single department or lab environment. |
| 2 Gigabit uplink ports | Provides faster connections to routers or core switches, reducing network bottlenecks. |
| VLAN support | Enables logical network segmentation for security and traffic management. |
| Auto-negotiation (speed/duplex) | Ensures compatibility with different devices and minimizes configuration errors. |
| Reliable Layer-2 switching | Provides fast packet forwarding with low latency for stable LAN performance. |

### 2.2.5 Access Point

Cisco Aironet 1832i

The Cisco Aironet 1832i is a business-class wireless access point designed to provide secure and reliable Wi-Fi connectivity. It allows wireless devices such as laptops to access the wired network without using physical cables.

| Features | Benefits |
| --- | --- |
| Dual-band Wi-Fi (2.4 GHz and 5 GHz) | Reduces congestion and improves wireless performance. |
| IEEE 802.11ac support | Enables high-speed wireless data transfer for modern applications. |
| Ethernet uplink port | Allows seamless integration into the wired network infrastructure. |
| Supports multiple simultaneous clients | Suitable for classroom and office environments with many users. |
| WPA/WPA2 security support | Protects wireless communications from unauthorized access. |

### 2.2.6 Server

Dell PowerEdge R710

The Dell PowerEdge R710 is an enterprise rack-mount server designed to operate continuously while hosting critical network services such as DNS, web hosting, file sharing, and DHCPv6.

| Features | Benefits |
| --- | --- |
| Dual Intel Xeon processors | Provides high computing performance for multiple services running simultaneously. |
| Large RAM capacity | Supports heavy workloads and multiple concurrent client connections. |
| RAID storage support | Protects data against disk failure and improves reliability. |
| Multiple Gigabit Ethernet interfaces | Enables redundancy and high network throughput. |
| Designed for 24/7 operation | Ensures high availability of network services. |

## 2.3 Network Software and Configurations

### 2.3.1 Network Software

Cisco Packet Tracer

Cisco Packet Tracer is a comprehensive network simulation software developed by Cisco for learning, teaching, and designing computer networks. It allows users to create, configure, and test network topologies in a virtual environment that closely resembles real-world networking scenarios. Through its graphical interface and built-in simulation capabilities, users can observe network behavior, troubleshoot connectivity issues, and validate configurations before real deployment. The software is widely used in academic institutions and professional training, especially for Cisco certification preparation.

Features and Benefits

| Features | Benefits |
| --- | --- |
| Graphical network simulation | Provides a visual platform for designing and understanding complex network topologies more easily. |
| Real-time and simulation modes | Allows testing of live network behavior or step-by-step packet flow analysis for troubleshooting and learning purposes. |
| Extensive device and protocol support | Supports routers, switches, servers, PCs, IoT devices, and protocols such as IPv6, OSPF, VLAN, DHCP, and DNS, enabling realistic enterprise network simulation. |
| IoT and programmability support | Enables advanced learning using IoT devices and scripting with Python and JavaScript for automation and smart network scenarios. |
| Alignment with Cisco certifications | Offers structured, scenario-based laboratories that support CCNA, CCNP, and other Cisco certification training. |

Windows Server 2025 Standard Edition

Windows Server 2025 Standard Edition is a server operating system developed by Microsoft for managing enterprise networks, applications, user authentication, and data storage. It provides a secure and scalable platform for hosting essential services such as DNS, DHCP, web servers, file servers, and directory services. In this project, Windows Server is suitable for simulating centralized network services and enterprise-level server management.

Features and Benefits

| Features | Benefits |
| --- | --- |
| Hybrid cloud integration | Supports seamless integration with cloud and on-premises environments, enabling organizations to extend services using Azure and Azure Arc. |
| Advanced security features | Provides hardware-based and software-based protection against cyberattacks, including enhanced Active Directory security and improved authentication mechanisms. |
| Modern and future-ready platform | Offers improved application compatibility, simplified updates, and easier system scalability for long-term use. |
| High performance networking and storage | Enhances data transfer speed, storage efficiency, and reduces downtime through features such as hot patching and optimized resource management. |

Paessler PRTG Network Monitor

Paessler PRTG is a professional network monitoring tool developed by Paessler GmbH in Germany. It is designed to continuously monitor network infrastructure, detect faults, and provide real-time performance statistics. PRTG is commonly used by network administrators to ensure high availability, reliability, and optimal utilization of network resources.

Features and Benefits

| Features | Benefits |
| --- | --- |
| Historical data storage and reporting | Enables long-term analysis of network performance trends, supporting capacity planning and optimization. |
| Multi-platform monitoring support | Allows administrators to access monitoring data from Windows, Linux, and macOS systems, increasing flexibility. |
| Comprehensive device monitoring | Monitors routers, switches, servers, bandwidth usage, and services in real time, ensuring early detection of network issues. |
| Web-based dashboards and alerts | Provides an intuitive interface with customizable dashboards and automated alerts, simplifying network management and decision-making. |

## 2.4 Cabling and Sockets Component

### 2.4.1 Cables

Cat6 UTP RJ45

The Category 6 (Cat6) unshielded twisted pair (UTP) straight-through cable is the standard Ethernet cable used to connect end devices such as PCs, servers, switches, routers, and wireless access points. It supports high-speed data transmission and is widely used in modern LAN infrastructures.

| Feature | Benefit |
| --- | --- |
| Category 6 (Cat6) standard (ANSI/TIA-568-C.2) | Supports data rates up to 1 Gbps (1000 Mbps) and bandwidth up to 250 MHz, ensuring fast and reliable LAN communication. |
| RJ-45 connectors (8P8C) | Universally compatible with Ethernet ports on PCs, switches, routers, servers, and access points. |
| Straight-through wiring (T568B–T568B) | Ideal for connecting different device types (PC→Switch, Switch→Router, Switch→Server), preventing wiring mismatches. |
| Unshielded Twisted Pair (UTP) design | Reduces electromagnetic interference and crosstalk, improving signal quality. |
| Copper conductors (24 AWG) | Provides stable electrical transmission and low signal loss over distances up to 100 meters. |
| Durable PVC outer jacket | Protects internal wires from physical damage and bending, increasing cable lifespan. |

Cisco CAB-SS-2626X

The Cisco Smart Serial DCE/DTE cable is a specialized WAN cable used to connect routers through serial interfaces. It is commonly used in enterprise networks to simulate leased lines and Frame Relay connections.

| Feature | Benefit |
| --- | --- |
| Smart Serial connector (Cisco proprietary) | Designed specifically for Cisco routers, ensuring secure and stable physical connection. |
| Supports DCE and DTE modes | Allows one router to provide clocking (DCE) while the other receives it (DTE), ensuring synchronized data transmission. |
| Supports Frame Relay and PPP/HDLC | Enables realistic simulation of WAN technologies used by ISPs and enterprises. |
| Low-latency serial communication | Suitable for long-distance routing and backbone links. |
| Lightweight and flexible cable design | Easy to install and manage in network racks and labs. |

NIM-2T (Router Module)

The Cisco Network Interface Module 2T (NIM-2T) is an expansion installed into the routers to provide additional serial communication ports. These ports are essential for simulating wide area network technologies such as point-to-point leased lines and Frame Relay connections.

| Features | Benefits |
| --- | --- |
| Two high-speed serial interfaces | Enables a single router to connect to multiple WAN links, supporting complex topologies and redundancy testing. |
| DCE and DTE mode support | Provides clock synchronization between routers, ensuring accurate and stable serial data transmission. |
| Frame Relay compatibility | Allows realistic simulation of enterprise WAN services using virtual circuits, improving understanding of service provider networking. |
| Modular plug-in design | Allows easy installation and removal without replacing the router, reducing hardware cost and improving scalability. |
| Full integration with Cisco IOS | Supports IPv6 addressing, OSPFv3 routing, and WAN encapsulation protocols for real-world configuration practice. |
| Compact hardware form factor | Saves internal router space while still delivering essential WAN connectivity features. |

## 2.5 Main Hour Requirements

| Main Hour Budget |  |
| --- | --- |
| Number of hours work a day | 8 hours |
| Total number of workers | 4 |
| Specific period: 10 November 2025 – 1 February | Public Holidays =3 days Weekends=24 days |
| Main Hour | 84 days -24days = 60days 60 days -3 days =57 working days Total man-hour calculation: 57 working hours x 8 hours/day x 4 workers = 1824 man-hours |

# 3.0 Network Topology

## 3.1 Logical Design

## 3.2 Physical Design

# 4.0 IPv6 Addressing Table

## 4.1 Lab1

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| Instructor(L1) | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC1_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC2_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC3_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC4_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC5_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC6_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC7_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC8_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC9_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC10_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC11_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC12_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC13_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC14_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC15_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC16_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC17_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC18_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC19_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC20_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC21_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |
| PC22_Lab1 | FastEthernet0 | 2026:A3F9:C21E:1:DHCPv6 assigned | FE80:1::1 |

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| Instructor(L2) | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC1_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC2_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC3_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC4_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC5_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC6_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC7_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC8_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC9_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC10_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC11_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC12_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC13_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC14_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC15_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC16_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC17_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC18_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC19_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC20_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC21_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |
| PC22_Lab2 | FastEthernet0 | 2026:A3F9:C21E:2:DHCPv6 assigned | FE80:2::1 |

## 4.2 Lab2

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| Sys.Adm1 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |
| Sys.Adm2 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |

## 4.3 System Adminstrator office

## 4.4 Administrator Office

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| PC_Intern1 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |
| PC_Intern2 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |
| PC_Intern3 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |
| PC_Intern4 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |
| PC_Intern5 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| PC_Staff1 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |
| PC_Staff2 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |
| PC_Staff3 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |
| PC_Staff4 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |
| PC_Staff5 | FastEthernet0 | 2026:A3F9:C21E:4:DHCPv6 assigned | FE80:4::1 |

## 4.5 Wireless device

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| Laptop1 | FastEthernet0 | 2026:A3F9:C21E:8:DHCPv6 assigned | FE80:8::1 |
| PC1 | FastEthernet0 | 2026:A3F9:C21E:8:DHCPv6 assigned | FE80:8::1 |

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| DNS Server | FastEthernet0 | 2026:A3F9:C21E:7::100/64 | FE80:7::1 |
| Internet Web Server | FastEthernet0 | 2026:A3F9:C21E:7::200/64 | FE80:7::1 |
| File Server | FastEthernet0 | 2026:A3F9:C21E:7::300/64 | FE80:7::1 |
| ISP Server | FastEthernet0 | 2026:A3F9:C21E:12::100/64 | FE80:12::1 |

## 4.6 Server

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| Lab Router | GigabitEthernet0/0/0 | 2026:A3F9:C21E:1::1/64 | FE80:1::1 |
|  | GigabitEthernet0/0/1 | 2026:A3F9:C21E:2::1/64 | FE80:2::1 |
|  | Serial0/1/0 | 2026:A3F9:C21E:3::1/64 | FE80:3::1 |
| Internal Router | GigabitEthernet0/0/0 | 2026:A3F9:C21E:4::1/64 | FE80:4::1 |
|  | GigabitEthernet0/0/1 | 2026:A3F9:C21E:5::1/64 | FE80:5::1 |
|  | Serial0/1/0 | 2026:A3F9:C21E:6::1/64 | FE80:6::1 |
| Main Router | GigabitEthernet0/0/0 | 2026:A3F9:C21E:7::1/64 | FE80:7::1 |
|  | GigabitEthernet0/0/1 | 2026:A3F9:C21E:8::1/64 | FE80:8::1 |
|  | Serial0/1/0 | 2026:A3F9:C21E:9::1/64 | FE80:9::1 |
|  | Serial0/1/1 | 2026:A3F9:C21E:10::1/64 | FE80:10::1 |
|  | Serial0/2/0 | 2026:A3F9:C21E:11::1/64 | FE80:11::1 |
| ISP Router | GigabitEthernet0/0/0 | 2026:A3F9:C21E:12::1/64 | FE80:12::1 |
|  | Serial0/1/0 | 2026:A3F9:C21E:11::2/64 | FE80:11::2 |

## 4.7 Router

| Device | Interface | IPv6 Address | Default Gateway |
| --- | --- | --- | --- |
| PC_Class1 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |
| PC_Class2 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |
| PC_Class3 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |
| PC_Class4 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |
| PC_Class5 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |
| PC_Class6 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |
| PC_Class7 | FastEthernet0 | 2026:A3F9:C21E:5:DHCPv6 assigned | FE80:5::1 |

## 4.8 Classrooms

# 5.0 Router Configuration

## 5,1 IPv6 Address Configuration

Lab Router

Router>enable Router#config terminal Router (config)#hostname Lab_Router Lab_Router(config)#ipv6 unicast-routing Lab_Router(config)#interface g0/0/0 Lab_Router(config-if)# ipv6 address 2026:A3F9:C21E:1::1/64 Lab_Router(config-if)# ipv6 address FE80:1::1 link-local Lab_Router(config-if)# :no shut Lab_Router(config-if)# exit Lab_Router(config)#interface g0/0/1 Lab_Router(config-if)# ipv6 address 2026:A3F9:C21E:2::1/64 Lab_Router(config-if)# ipv6 address FE80:2::1 link-local Lab_Router(config-if)# :no shut Lab_Router(config-if)# exit Lab_Router(config)# interface s0/1/0 Lab_Router(config-if)# ipv6 address 2026:A3F9:C21E:3::1/64 Lab_Router(config-if)# ipv6 address FE80:3::1 link-local Lab_Router(config-if)# no shut Lab_Router(config-if)#exit Lab_Router(config)#exit Lab_Router#copy run start

Internal Router

Router>enable Router#config terminal Router (config)#hostname Internal_Router Internal_Router(config)#ipv6 unicast-routing Internal_Router(config)#interface g0/0/0 Internal_Router(config-if)# ipv6 address 2026:A3F9:C21E:4::1/64 Internal_Router(config-if)# ipv6 address FE80:4::1 link-local Internal_Router(config-if)# :no shut Internal_Router(config-if)# exit Internal_Router(config)#interface g0/0/1 Internal_Router(config-if)# ipv6 address 2026:A3F9:C21E:5::1/64 Internal_Router(config-if)# ipv6 address FE80:5::1 link-local Internal_Router(config-if)# :no shut Internal_Router(config-if)# exit Internal_Router(config)# interface s0/1/0 Internal_Router(config-if)# ipv6 address 2026:A3F9:C21E:6::1/64 Internal_Router(config-if)# ipv6 address FE80:6::1 link-local Internal_Router(config-if)# no shut Internal_Router(config-if)#exit Internal_Router(config)#exit Internal_Router#copy run start

Main Router

Router>enable Router#config terminal Router (config)#hostname Main_Router Main_Router(config)#ipv6 unicast-routing Main_Router(config)#interface g0/0/0 Main_Router(config-if)# ipv6 address 2026:A3F9:C21E:7::1/64 Main_Router(config-if)# ipv6 address FE80:7::1 link-local Main_Router(config-if)# :no shut Main_Router(config-if)# exit Main_Router(config)#interface g0/0/1 Main_Router(config-if)# ipv6 address 2026:A3F9:C21E:8::1/64 Main_Router(config-if)# ipv6 address FE80:8::1 link-local Main_Router(config-if)# :no shut Main_Router(config-if)# exit Main_Router(config)# interface s0/1/0 Main_Router(config-if)# ipv6 address 2026:A3F9:C21E:9::1/64 Main_Router(config-if)# ipv6 address FE80:9::1 link-local Main_Router(config-if)# no shut Main_Router(config-if)#exit Main_Router(config)# interface s0/1/1 Main_Router(config-if)# ipv6 address 2026:A3F9:C21E:10::1/64 Main_Router(config-if)# ipv6 address FE80:10::1 link-local Main_Router(config-if)# no shut Main_Router(config-if)#exit Main_Router(config)# interface s0/2/0 Main_Router(config-if)# ipv6 address 2026:A3F9:C21E:11::1/64 Main_Router(config-if)# ipv6 address FE80:11::1 link-local Main_Router(config-if)# no shut Main_Router(config-if)#exit Main_Router(config)#exit Main_Router#copy run start

ISP Router

Router>enable Router#config terminal Router (config)#hostname ISP_Router ISP_Router(config)#ipv6 unicast-routing ISP_Router(config)#interface g0/0/0 ISP_Router(config-if)# ipv6 address 2026:A3F9:C21E:12::1/64 ISP_Router(config-if)# ipv6 address FE80:12::1 link-local ISP_Router(config-if)# :no shut ISP_Router(config-if)# exit ISP_Router(config)# interface s0/1/0 ISP_Router(config-if)# ipv6 address 2026:A3F9:C21E:11::2/64 ISP_Router(config-if)# ipv6 address FE80:11::2 link-local ISP_Router(config-if)# no shut ISP_Router(config-if)#exit ISP_Router(config)#exit ISP_Router#copy run start

## 5.2 DHCPv6 Configuration

DHCPv6 stateful Pool

DNS server=2026:A3F9:C21E:7::100/64

Lab Router

Lab_Router>en Lab_Router# conf t Lab_Router(config)#ipv6 dhcp pool STATEFUL_1 Lab_Router(config-dhcpv6)#address prefix 2026:A3F9:C21E:1::1/64 Lab_Router(config-dhcpv6)#dns-server 2026:A3F9:C21E:7::100 Lab_Router(config-dhcpv6)#domain-name FIST_Lab.com Lab_Router(config-dhcpv6)#exit Lab_Router(config)#ipv6 dhcp pool STATEFUL_2 Lab_Router(config-dhcpv6)#address prefix 2026:A3F9:C21E:2::1/64 Lab_Router(config-dhcpv6)#dns-server 2026:A3F9:C21E:7::100 Lab_Router(config-dhcpv6)#domain-name FIST_Lab.com Lab_Router(config-dhcpv6)#exit Lab_Router(config)#int g0/0/0 Lab_Router(config-if)#ipv6 nd managed-config-flag Lab_Router(config-if)#ipv6 dhcp server STATEFUL_1 Lab_Router(config-if)#exit Lab_Router(config)#int g0/0/1 Lab_Router(config-if)#ipv6 nd managed-config-flag Lab_Router(config-if)#ipv6 dhcp server STATEFUL_2 Lab_Router(config-if)#exit Lab_Router(config)#exit Lab_Router#copy run start

Internal_Router

Internal_Router>en Internal_Router# conf t Internal_Router(config)#ipv6 dhcp pool STATEFUL_4 Internal_Router(config-dhcpv6)#address prefix 2026:A3F9:C21E:4::1/64 Internal_Router(config-dhcpv6)#dns-server 2026:A3F9:C21E:7::100 Internal_Router(config-dhcpv6)#domain-name FIST_Internet.com Internal_Router(config-dhcpv6)#exit Internal_Router(config)#ipv6 dhcp pool STATEFUL_5 Internal_Router(config-dhcpv6)#address prefix 2026:A3F9:C21E:5::1/64 Internal_Router(config-dhcpv6)#dns-server 2026:A3F9:C21E:7::100 Internal_Router(config-dhcpv6)#domain-name FIST_Internet.com Internal_Router(config-dhcpv6)#exit Internal_Router(config)#int g0/0/0 Internal_Router(config-if)#ipv6 nd managed-config-flag Internal_Router(config-if)#ipv6 dhcp server STATEFUL_4 Internal_Router(config-if)#exit Internal_Router(config)#int g0/0/1 Internal_Router(config-if)#ipv6 nd managed-config-flag Internal_Router(config-if)#ipv6 dhcp server STATEFUL_5 Internal_Router(config-if)#exit Internal_Router(config)#exit Internal_Router#copy run start

Main_Router

Main_Router>en Main_Router# conf t Main_Router(config)#ipv6 dhcp pool STATEFUL_7 Main_Router(config-dhcpv6)#address prefix 2026:A3F9:C21E:7::1/64 Main_Router(config-dhcpv6)#dns-server 2026:A3F9:C21E:7::100 Main_Router(config-dhcpv6)#domain-name FIST_Main.com Main_Router(config-dhcpv6)#exit Main_Router(config)#ipv6 dhcp pool STATEFUL_8 Main_Router(config-dhcpv6)#address prefix 2026:A3F9:C21E:8::1/64 Main_Router(config-dhcpv6)#dns-server 2026:A3F9:C21E:7::100 Main_Router(config-dhcpv6)#domain-name FIST_Main.com Main_Router(config-dhcpv6)#exit Main_Router(config)#int g0/0/0 Main_Router(config-if)#ipv6 nd managed-config-flag Main_Router(config-if)#ipv6 dhcp server STATEFUL_7 Main_Router(config-if)#exit Main_Router(config)#int g0/0/1 Main_Router(config-if)#ipv6 nd managed-config-flag Main_Router(config-if)#ipv6 dhcp server STATEFUL_8 Main_Router(config-if)#exit Main_Router(config)#exit Main_Router#copy run start

## 5.3 OSPF configuration

Lab Router

Lab_Router>en Lab_Router# conf t Lab_Router(config)#ipv6 router ospf 10 Lab_Router(config-rtr)#router-id 1.1.1.1 Lab_Router(config-rtr)#exit Lab_Router(config)#int g0/0/0 Lab_Router(config-if)#ipv6 ospf 10 area 0 Lab_Router(config-if)#no shut Lab_Router(config-if)#exit Lab_Router(config)#int g0/0/1 Lab_Router(config-if)#ipv6 ospf 10 area 0 Lab_Router(config-if)#no shut Lab_Router(config-if)#exit Lab_Router(config)#int s0/1/0 Lab_Router(config-if)#ipv6 ospf 10 area 0 Lab_Router(config-if)#no shut Lab_Router(config-if)#exit Lab_Router(config)#exit Lab_Router#copy run start

Internal_Router

Internal_Router>en Internal_Router# conf t Internal_Router(config)#ipv6 router ospf 10 Internal_Router(config-rtr)#router-id 2.2.2.2 Internal_Router(config-rtr)#exit Internal_Router(config)#int g0/0/0 Internal_Router(config-if)#ipv6 ospf 10 area 0 Internal_Router(config-if)#no shut Internal_Router(config-if)#exit Internal_Router(config)#int g0/0/1 Internal_Router(config-if)#ipv6 ospf 10 area 0 Internal_Router(config-if)#no shut Internal_Router(config-if)#exit Internal_Router(config)#int s0/1/0 Internal_Router(config-if)#ipv6 ospf 10 area 0 Internal_Router(config-if)#no shut Internal_Router(config-if)#exit Internal_Router(config)#exit Internal_Router#copy run start

Main_Router

Main_Router>en Main_Router# conf t Main_Router(config)#ipv6 router ospf 10 Main_Router(config-rtr)#router-id 3.3.3.3 Main_Router(config-rtr)#exit Main_Router(config)#int g0/0/0 Main_Router(config-if)#ipv6 ospf 10 area 0 Main_Router(config-if)#no shut Main_Router(config-if)#exit Main_Router(config)#int g0/0/1 Main_Router(config-if)#ipv6 ospf 10 area 0 Main_Router(config-if)#no shut Main_Router(config-if)#exit Main_Router(config)#int s0/1/0 Main_Router(config-if)#ipv6 ospf 10 area 0 Main_Router(config-if)#no shut Main_Router(config-if)#exit Main_Router(config)#int s0/1/1 Main_Router(config-if)#ipv6 ospf 10 area 0 Main_Router(config-if)#no shut Main_Router(config-if)#exit Main_Router(config)#int s0/2/0 Main_Router(config-if)#ipv6 ospf 10 area 0 Main_Router(config-if)#no shut Main_Router(config-if)#exit Main_Router(config)#exit Main_Router#copy run start

ISP_Router

ISP_Router>en ISP_Router# conf t ISP_Router(config)#ipv6 router ospf 10 ISP_Router(config-rtr)#router-id 4.4.4.4 ISP_Router(config-rtr)#exit ISP_Router(config)#int g0/0/0 ISP_Router(config-if)#ipv6 ospf 10 area 0 ISP_Router(config-if)#no shut ISP_Router(config-if)#exit ISP_Router(config)#int s0/1/0 ISP_Router(config-if)#ipv6 ospf 10 area 0 ISP_Router(config-if)#no shut ISP_Router(config-if)#exit ISP_Router(config)#exit ISP_Router#copy run start

# 6.0 Frame Relay Configuration

| Frame Relay |  |  |  |
| --- | --- | --- | --- |
| From | Sublink | To Port | Sublink |
| Serial 0 | Main Router | Serial 1 | ISP Router |

Main_Router

Main_Router>en Main_Router# conf t Main_Router(config)#int s0/2/0 Main_Router(config-if)#encapsulation frame-relay Main_Router(config-if)#frame-relay map ipv6 2026:A3F9:C21E:11::2 100 Main_Router(config-if)#frame-relay map ipv6 FE80:11::2 100 broadcast Main_Router(config)#end Main_Router#copy run start

ISP_Router

ISP_Router>en ISP_Router# conf t ISP_Router(config)#int s0/1/0 ISP_Router(config-if)#encapsulation frame-relay ISP_Router(config-if)#frame-relay map ipv6 2026:A3F9:C21E:11::1 101 ISP_Router(config-if)#frame-relay map ipv6 FE80:11::1 101 broadcast ISP_Router(config)#end ISP_Router#copy run start

# 7.0 Access Control List Configuration

Main_Router

Main_Router(config)#ipv6 access-list BLOCK_HTTP Main_Router(config-ipv6-acl)#deny tcp any host 2026:A3F9:C21E:12::100 eq www Main_Router(config-ipv6-acl)#deny tcp any host 2026:A3F9:C21E:12::100 eq 443 Main_Router(config-ipv6-acl)#permit ipv6 any any Main_Router(config-ipv6-acl)#exit Main_Router(config)#int g0/0/1 Main_Router(config-if)#ipv6 traffic-filter BLOCK_HTTP in Main_Router(config-if)#exit Main_Router(config)# Main_Router#copy run start

Internal_Router

Internal_Router(config)#ipv6 access-list BLOCK_HTTP Internal_Router(config-ipv6-acl)#deny tcp any host 2026:A3F9:C21E:12::100 eq www Internal_Router(config-ipv6-acl)#deny tcp any host 2026:A3F9:C21E:12::100 eq 443 Internal_Router(config-ipv6-acl)#permit ipv6 any any Internal_Router(config-ipv6-acl)#exit Internal_Router(config)#int g0/0/1 Internal_Router(config-if)#ipv6 traffic-filter BLOCK_HTTP in Internal_Router(config-if)#exit Internal_Router(config)#exit Internal_Router#copy run start

Main_Router

Main_Router(config)#ipv6 access-list BLOCK_HTTP Main_Router(config-ipv6-acl)#deny tcp any host 2026:A3F9:C21E:12::100 eq www Main_Router(config-ipv6-acl)#deny tcp any host 2026:A3F9:C21E:12::100 eq 443 Main_Router(config-ipv6-acl)#permit ipv6 any any Main_Router(config-ipv6-acl)#exit Main_Router(config)#int g0/0/1 Main_Router(config-if)#ipv6 traffic-filter BLOCK_HTTP in Main_Router(config-if)#exit Main_Router(config)#exit Main_Router#copy run start

# 8.0 Server Configuration

## 8.1 Web Server Configuration

Ipv6 address:2026:A3F9:C21E:7::200/64

Default gateway:FE80:7::1

DNS Server:2026:A3F9:C21E:7::100

Edit index.html

## 8.2 DNS Server Configuration

Ipv6 address:2026:A3F9:C21E:7::100/64

Default gateway:FE80:7::1

DNS server:2026:A3F9:C21E:7::100

Turn off HTTP and HTTPS services

Turn on DHCPv6 service

Turn on DNS service

- Name:www.mmu.edu.my

Type:AAAAA record

Address:2026:A3F9:C21E:7::200

- Name:internetserver.com

Type:AAAAA record

Address:2026:A3F9:C21E:7::100

- Name:ftp.com

Type:AAAAA record

Address:2026:A3F9:C21E:7::300

## 8.3 File Server Configuration

Ipv6 address:2026:A3F9:C21E:7::300/64

Default gateway:FE80:7::1

DNS Server:2026:A3F9:C21E:7::100

Turn on HTTP and HTTPS services

Turn off DHCPv6 service

Turn off DNS service

Turn on FTP service

Username: admin

Password admin123

Select Write,Read, Delete, Rename and List and click “Add”

## 8.4 ISP Server Configuration

Ipv6 address:2026:A3F9:C21E:12::100/64

Default gateway:FE80:12::1

DNS Server:2026:A3F9:C21E:7::100

## 8.5 Cloud

# 9.0 Verify Connectivity

## 9.1 Ping(ICMP)

1.Ping from Instructor L1 to PC3_Lab2

2.Ping from Instructor L2 to PC8_Lab1

3.Ping from PC18_Lab2 to PC13_Lab1

4.Ping PC14_Lab1 to File Server

5. Ping from PC8_Lab2 to Staff3

6.Ping from Intern1 to Staff1

7.Ping from Sys Adm2 to Class 3

8.Ping from Class 7 to Web Server

9. Ping from Intern 4 to File Server

10. Ping from Staff 5 to DNS Server

## 9.2 Web Browser

1. Access www.mmu.edu.my from PC21_Lab1

2.Access www.mmu.edu.my from PC18_Lab2

3.Access www.mmu.edu.my from Staff2

4.Access www.mmu.edu.my from Class1

5.Access www.mmu.edu.my  from Intern3

## 9.3 File Server

Administrator office

Create a text file in Staff2

Login into ftp server in Staff2 and upload the text file(Hello.txt)

Enter the ftp server and retrieve the test file for PC12_Lab2

Go to “Text Editor” and click on “File” and “open” button. Select “Hello.txt”

Wireless Network

Create a text file in Wireless Laptop

Login into ftp server in the Wireless Laptop and upload the text file (test.txt)

Enter ftp server and get the file from Class 3.

Go to text editor and see the file.

## 9.4 Access control

1.PC3-Lab1 can access to the internet.

2.PC3-Lab2 can access to the internet.

3.Staff 5 in admin office cannot access to the internet.

4.Intern1 in admin office cannot access to the internet.

5.Class1 can access to the internet.

6.Sys adm1 access to the internet.

# 10 .0 Reference

- Configure DHCPv6 (variation in Packet Tracer): https://youtu.be/uCQw8mmQ05A

- Configure a Wireless Network: https://youtu.be/F5A9cG22Sfw

- DNS Server Cisco Packet Tracer: https://youtu.be/DfaN-YTDNyE

- https://www.cisco.com/c/en/us/support/routers/4331-integrated-services-router-isr/model.html?dtid=osscdc000283

- https://i.dell.com/sites/doccontent/business/smb/merchandizing/en/Documents/OptiPlex_7010_Technical_Guidebook_1.pdf

- https://www.dell.com/en-us/shop/desktop-computers/optiplex-tower/spd/optiplex-7010-desktop

- https://www.dell.com/support/manuals/en-us/latitude-e6430s/late6430s_om-v1/technical-specifications?guid=guid-689ea094-1adc-4409-9288-0bfdfbcd78a1&lang=en-us

- Configure Frame-relay: https://www.computernetworkingnotes.com/ccna-study-guide/how-to-configure-frame-relay-step-by-step-guide.html

- https://www.paessler.com/prtg/features

- https://www.microsoft.com/en-us/evalcenter/evaluate-windows-server-2025

- https://i.dell.com/sites/csdocuments/Shared-Content_data-Sheets_Documents/en/R710-SpecSheet.pdf

- https://www.dell.com/support/product-details/en-us/product/optiplex-7010/overview

- https://www.cisco.com/c/en/us/support/docs/routers/7000-series-routers/12222-cables.html?utm_source=chatgpt.com